GZTpSelectedReader_updated11JUL16_better_Juan

- Select the paths of the IV and IZ files with the script: ‘setIVIZdir’.

- You can choose 1,2 or 3 files:

-Just the IZ.

-The IZ, the calibration from .txt and the temperature file .tem.

The program saves a structure with all the data, you can call it by:

Curva = get(num.fig , ‘userdata’)

The data of the thermopower is saved in:

Curva.joinedDataTpPol1.Thermopower

Here there are 8 different data, this is because the program calculates the Vth for 8 different voltage amplitudes and it checks which one is the best (don’t know how).

The Offsets are calculated in two parts:

-IOffset: You can choose if you want to calculate it from last IV curves (if you choose this option you have to be sure that the button ‘Shot las IVs’ in the Zetaprobe program is check) of by measuring the end of the IZ curves. You choose this at the beginning of the program in the command ‘ExtraIVs=1 or 0’. This is saved in ‘IoffIVcell’ for the last IVs or in ‘IoffGZcell’ for the end of the IZ curves.

-VOffset: This is calculated with the .txt program. In the script IvTpOffset the program reads the .txt file and save it in a matrix called ‘datoffs’. From here it saves it in the final structure at the first position with the following name:

Curva(1).VeOffsets

The mean value of the Ve (this value changes with the Zetaprobe version, see info) calculated in the Zetaprobe is

Curva(1).VeOffsets.MeanOffset

Finally the offsets are included in the curves in the function ‘getIVpoints’ at xxof and yyof.

The IV curves in the IVs button have the Ioffset included in the plots but not the Voffset